Preparation of the initial wave packet for the quantum interferometer-gravimeter

Adamov Kirill Nikolaevich1, 2, Kapusta Dmitry Nikolaevich1, 2, Bonert Anatoly Ernstovich1, Goncharov Andrey Nikolaevich1, 2, 3

1Institute of Laser Physics SB RAS, Novosibirsk, Russia

2Novosibirsk State University, Novosibirsk, Russia

3Novosibirsk State Technical University, Novosibirsk, Russia

e-mail: ofadamovkirill@gmail.com

We investigated the preparation of ultracold 87Rb atoms in a magnetically insensitive quantum state for application in an atomic interferometer-based gravimeter. Numerical modeling of optical pumping of rubidium atoms to the state ∣F=1, m​=0⟩ was performed using the ∣F=1⟩↔∣F′=0⟩ transition by solving rate equations that describe the population distribution among magnetic sublevels of the ground state. Simulation results demonstrate high efficiency of the selected pumping scheme (~100%) without significant heating of the atomic cloud. Implementation of this method enables preparation of 87Rb atoms in the magnetically insensitive state ∣F=1, m​=0⟩ with efficiency exceeding 95% and temperature increase of around 1 μK. The use of optical pumping results in more than a fourfold improvement in gravimeter sensitivity, achieving a sensitivity of 5.2 mGal/√Hz. Additionally, we investigated vibration noise compensation using a post-correction signal method with an accelerometer. Implementation of this technique improved the output metrological characteristics of the gravimeter by a factor of two.