Preparation of the initial wave packet for the quantum interferometer-gravimeter

Adamov Kirill Nikolaevich1, 2, Kapusta Dmitry Nikolaevich1, 2, Bonert Anatoly Ernstovich1, Goncharov Andrey Nikolaevich1, 2, 3

1Institute of Laser Physics SB RAS, Novosibirsk, Russia

2Novosibirsk State University, Novosibirsk, Russia

3Novosibirsk State Technical University, Novosibirsk, Russia

e-mail: ofadamovkirill@gmail.com

This work was aimed at improving metrological characteristics of the developed atomic interferometer-based gravimeter. We investigated a preparation of ultracold 87Rb atoms in a magnetically insensitive quantum state. Numerical modeling of optical pumping of rubidium atoms to the ∣F=1, m​=0⟩ state was performed using the ∣F=1⟩↔∣F′=0⟩ transition by solving rate equations that describe the population distribution among magnetic sublevels of the ground state. Simulation results demonstrate the high efficiency of the selected pumping scheme (~100%) without significant heating of the atomic cloud. Implementation of this method enables preparation of 87Rb atoms in the magnetically insensitive ∣F=1, m​=0⟩ state with efficiency >95% and a temperature increase of only 1 μK. The use of optical pumping results in more than a fourfold improvement in the gravimeter sensitivity, achieving 5.2 mGal/√Hz. A compensation of the gravimeter vibration noise was performed using a post-correction method together with an accelerometer data. Implementation of this technique improved the output metrological characteristics of the gravimeter by a factor of two.