Preparation of the initial wave packet for the quantum interferometer-gravimeter

Adamov Kirill Nikolaevich1, 2, Kapusta Dmitriy Nikolaevich1, 2, Bonert Anatoly Ernstovich1, Goncharov Andrey Nikolaevich1, 2, 3

1Institute of Laser Physics SB RAS, Novosibirsk, Russia

2Novosibirsk State University, Novosibirsk, Russia

3Novosibirsk State Technical University, Novosibirsk, Russia

e-mail: ofadamovkirill@gmail.com

The precise measurement of gravitational acceleration represents a significant challenge in modern science and technology. High-sensitivity and high-precision gravimeters, as instruments for detecting gravitational acceleration, play a crucial role both in fundamental research and in addressing a wide range of applied tasks. Of particular importance are absolute quantum gravimeters, which offer several key advantages: absence of drift, lack of moving mechanical parts, high measurement speed, and the potential for integrating gravimeter, gyroscope, and accelerometer functionalities within a single device.

The accuracy of a quantum gravimeter depends on the characteristics of the ultracold 87Rb atom source, including the atom number in the cloud, their temperature, the efficiency of preparing the initial atomic wave packet, and other related factors. A significant influence on sensitivity and stability is exerted by various sources of noise, with vibrational noise contributing the most. To suppress vibrational noise, a vibration-isolation platform is employed in conjunction with a signal post-correction system [2], or alternatively, an active compensation system for vibrational disturbances is used.

In this work, we investigate the preparation of ultracold 87R atoms in a magnetically insensitive quantum state for application in an atomic interferometer-based gravimeter. Numerical modeling of the optical pumping of rubidium atoms to the ∣F=1,m​=0⟩ state was performed using the ∣F=1⟩↔∣F′=0⟩ transition by numerically solving rate equations that describe the population distribution among the magnetic sublevels of the ground state [1]. The simulation results demonstrated high efficiency of the selected pumping scheme (about ~100%) without significant heating of the atomic cloud. Implementation of this methodic enabled the preparation of 87Rb atoms in the magnetically insensitive ∣F=1,mF​=0⟩ state with an efficiency exceeding 95% (see Fig. 1(a)) and a temperature increase of less than 1 μK. The use of optical pumping resulted in more than a fourfold improvement in gravimeter sensitivity, achieving a sensitivity of 5.2 mGal/√Hz (see Fig. 1(b)).

Fig. 1. The dependence of the atomic population before and after optical pumping on the detuning relative to the hourly transition (a) and the dependence of the standard deviation σg gravitational acceleration on the averaging time after optical pumping (b).

An investigation of vibration noise compensation using a post-correction signal method with an accelerometer was also conducted. The implementation of this technique led to improved output metrological characteristics of the gravimeter.

[1] F. Antoche, A. Kastberg Simplified approach for quantitative calculations of optical pumping // European Journal of Physics. - 2017. - №4. - С. 045703.

[2] J. Guo, S. Ma, C. Zhou, J. Liu, B. Wang, D. Pan, H. Mao Vibration compensation for a vehicle-mounted atom gravimeter // IEEE Sensors Journal. - 2022. - №13. - С. 12939-12946.